| PIPING |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1 | 1190-L-DW-0400 - EQUIPMENT LAYOUT - OVERALL PLOT PLAN and 1190-L-DW-434 - EQUIPMENT LAYOUT -STORAGE AND MAINTAINANCE AREA |  | Storage and maintenance area | In overall plot plan the facility is shown in a different location with respect to the location shown in equipment layout drawing, please confirm the actual location. | Bidder to follow the location as per Overall plot plant |
| 2 | 1190-L-DW-0400 - EQUIPMENT LAYOUT - OVERALL PLOT PLAN and 1190-M-LL-0500 Equipments List Rev.D (02.July.2014) |  | Water Supply system and storage tank | In overall plot plan it shows that 2 nos of Water Supply system - storage tank but in 1190-M-LL-0500 Equipments List Rev.D it shows only 1 number of Water Supply system - storage tank, please clarify. | It should be 1 System with 2 storage tanks. No mistake in equipment list. It says 2 tanks (1 for industrial water, 1 for potable water). Please re-check. |
| 3 | 1190-L-DW-0400 - EQUIPMENT LAYOUT - OVERALL PLOT PLAN and 1190-S-MT-0600 Civil and structural Material take off |  | Watch Tower | In overall plot plan and in Civil and structural Material take off it shows that 3nos watch tower Numbering WT-1, WT-2, WT-4 are considered. Please highlight on WT-3 if any. | There is no WT-3 . Bidder to follow as per overall layout( Only 3 watch towers to be considered). |
| 4 | 1190-L-DW-0400 - EQUIPMENT LAYOUT - OVERALL PLOT PLAN |  | Boiler | In future area it shows a Boiler. But it is not found in equipment list or any civil-structural scope of work . Please clarify the same. | Boiler is not in EPC scope. |
| 5 | 1190-L-DW-0400 - EQUIPMENT LAYOUT - OVERALL PLOT PLAN |  | Road | Tender does not give any specific data for specification of road whether it is a bituminous road or anything else. Please mention the specification. | Type of road is designed as Bituminous. Bidder to refer cl 1.2 of 1190-S-GD-0600. |
| 6 |  |  | Equipment Layout - OISD & NFPA compliance | Please confirm that the locations of equipments as shown in the Equipment Layouts are as per OISD and NFPA codes. | It is as per OISD and NFPA |