| PROCESS |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1 | Process Simulation Report-Doc No : 1190 -P-RP-0100 | Sl. No. 8 | This clause states that ‘Separation system will be two trains of 2x1800 KLPD and Stabilization system will be two trains of 3600 KLPD’ | As per PFDs and Equipment list, there are 4 indirect bath heaters ( 2 each in train A & B respectively) each of capacity 1800 KLPD. Which means, separation system is with 2x3600 KLPD capacities. Thus, we presume that for separation as well as stabilization, there will be 2 trains , each with 3600 KLPD capacity. Please confirm. | ALL the HP wells are routed to indirect bath heater-A & all the LP wells are routed to indirect bath heater-B. Both were operating under different pressures, so there are 2 trains (1 train operating & 1 train standby) of 2x1800 KLPD crude preheating & separation system. The stabilization system for both the trains operates under similar pressure, so there are 2 trains of 3600 KLPD. Bidder understanding is correct. |
| 2 | Process design Basis | Clause 4.1,4.2,4.3 | The referred clauses specify the reservoir data such as gas composition, crude oil data and crude oil composition. | Bidder has considered the incoming well fluid data as per referred clauses as basis for performance. Please confirm. | Bidder to consider all the reservoir data (clause 4.1, 4.2, 4.3, 4.4, 4.5.1, 4.5.2, 4.5.3, 4.6, 4.7, 4.8, 4.9, 4.10, 4.11) provided in process design basis. |
|  |  | Performance Guarantees | a. Overall system performance guarantee b. Individual process equipment/process package performance guarantees. | Please provide criteria/values for the performance guarantees. | It has been already provided in process design basis (Clause-5.0) & process description (refer EPC Tender volume-2). |
| 3 | Mechanical RFQ - Indirect Bath Heaters Doc. No.: 1190-M-TS-0501-01 |  |  | Datasheet is not provided in this RFQ . Request to provide the same. | Datasheet will be provided as an addendum |
| 4 | Mechanical RFQ - Gas Compression System Doc. No.: 1190-M-TS-0501-03 |  |  | Datasheet is not provided in this RFQ . Request to provide the same. | Datasheet will be provided as an addendum |
| 5 | Mechanical RFQ - Instrument & Utility Air compressor-Doc. No.: 1190-M-TS-0505-01 |  |  | Datasheet is not provided in this RFQ . Request to provide the same. | Datasheet will be provided as an addendum |
| 6 | Mechanical RFQ - Instrument Air dryer package-Doc. No.: 1190-M-TS-0505-02-RFQ-IA |  |  | Datasheet is not provided in this RFQ . Request to provide the same. | Datasheet will be provided as an addendum |
| 7 | - PFD No.1190-P-DW-0111 - PID No. 1190-P-DW-0701/0702 | Hot oil supply /return lines | - In referred PFD, Hot oil supply lines are shown to 4 users ie., Crude oil test tank, untreated crude storage tank ,crude oil heater and crude oil storage tanks. However in referred PID, , Hot oil supply lines are shown to 3 users ie., Untreated crude storage tank ,crude oil heater and crude oil storage tanks. - In referred PFD, Hot oil return lines are shown from 5 users ie. Crude oil test tank, untreated crude storage tank ,crude oil heater , crude oil storage tanks and Test tank. However in referred PID, Hot oil return lines are shown from 3 users ie., Untreated crude storage tank ,crude oil heater and crude oil storage tanks. - For test tank, referred in point b above, no reference /details available. | Please clarify about the users for hot oil. Also provide details of Test tank indicated in point c. | a. Hot oil is supplied to 1) crude oil heater (HBG-A-1020), 2)crude storage tank (BBJ-1060 A/B/C), 3) untreated crude storage tank (BBJ-1090), 4) crude oil test tank (BBJ-1552) and Bidder to consider hot oil for skim oil tank (ABJ-1120), hot oil to skim oil tank will not affect the hot oil requirement because the requirement can be accommodated in the margin (30%) we have considered. Including skim oil tank hot oil requirement we have 10% excess hot oil than required. Refer utility consumption schedule (1190-P-SH-0100). b. Hot oil return lines are from the same above said vessels. The PFDs and PIDs will be revised accordingly. c. This is typo error; bidder to update. |
| 8 | Functional Specification for Packaged Equipment - 1190-M-FS-501- |  |  | This specification is not provided. Please provide the same. | Datasheet will be provided as an addendum |
| 9 | Datasheets for Inlet separators/ LP separators |  |  | Min/max values for temp./pressure are not indicated in datasheet, which can vary the performance of separators. Please provide the same. | The inlet separator-A will be operated at 20 Kg/cm2g and inlet separator-B will be operated at 9 Kg/cm2g. The pressure reduction of well fluids is by PCV upstream of indirect bath heaters for the corresponding trains. Hence there are no min/max pressure available at HP & LP separators. Bidder to comply. |
| 10 | Process Description | a.Clause 8 – Instrument air/ Utility air system b. Clause 9 – Nitrogen supply c. PID No. 1190-P-DW-1603 | a. In clause 8, it is mentioned that part of utility air is distributed from utility air receiver to nitrogen generation package . b. In clause 9, it is mentioned that nitrogen will be provided with the help of nitrogen cylinders. Also in referred PID, nitrogen supply is shown by cylinders. | Please confirm method of nitrogen supply. | a.There is no nitrogen generation package in OCS and distribution of utility air to nitrogen generation package is an error, bidder to correct. b. it is confirmed that nitrogen is supplied with the help of nitrogen bottles. |