| PROCESS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Part - II: Technical Section, Section -I- Process |  | (Doc No - TCE.7547A-CH-E-PDS-8721, Rev R1) |  |  | Datasheet for Plate heat exchanger (E-002), page no 1 of 2 |  |  | TRS-02(Sr No 55 Process) |  |  | Capacity |  |  | Through TRS-02 (Sr No 55 Process), OWNER confirmed that “Kindly consider 210 m3/hr as a circulation flow rate for cooling tower.” BIDDER understands that heat exchanger E-002 to be hydraulically designed for 13000 KLPD. However the heat duty shall be based on cooling water flowrate of 210 m3/hr and cooling water delta T as 4 Deg C. Hence heat duty of E-002 to be revised from 4.8 Mkcal/hr to 0.84 Mkcal/hr. Request OIL concurrence. |  |  |  | The heat duty to be performed by E-002 will be dependent the temperature of the Dry Crude Oil from EET outlet specified by the EET vendor & the temperature required at the down stream of E-002. The bidder shall design E-002 based on this condition without altering the cooling water flowrate. |  |  |
|  |  | Technical |  | - |  |  | TQ Replies |  |  | TRS-03 (Sr no 04 Electrical) |  |  | VFD |  |  | Through TRS-03 (Sr No 04 Electrical), OWNER confirmed that “As VFD is not required; the specification for the same is not included.” However following equipment are considered with VFD as per Bid P&IDs. 1.Ultra filtration Feed Pump for ETP and OWTP 2.Ultra filtration backwash pump for ETP and OWTP 3.Filter backwash pumps for OWTP 4.Chemical Additive Dosing System Pumps 5. FD fan in boiler (as per Boiler specification). (As per vendor feedback VFD is required for low NOx burner with O2 trimming) BIDDER understands that the above equipment can be considered with VFD as per bid requirement. Vendor’s specifications will be followed for the same. There is no obligation to follow OIL/TCE VFD specifications (since not provided in bid). |  |  |  | Refer Corrigendum |  |  |
|  |  | Technical |  | Part - II: Technical Section, Section -II- Process , (Spec No - TCE.7547A-CH-300-CN-02, Rev R0) |  |  | Concept note for ETP, page no 3 of 14 |  |  | 3.2 |  |  | Location of Oxygen scavenger |  |  | Section 3.2 of Concept note for ETP indicates required treated water characteristics at the outlet of Ultra-Filtration. Further it has been indicated that “In case of DO > 0.02 mg/l at the outlet of ETP, oxygen scavenger can be used to meet desired outlet DO parameter; the same shall be taken care by EPC contractor”. OWNER to please note that even if the water quality is as per specifications at the outlet of UF (with oxygen scavenger dosing), there could be chances of oxygen ingress in T-013/014 since the said tank is open to atmosphere. In view of above OWNER to confirm the location of dosing of oxygen scavenger i.e. to be consider at the outlet of UF (of ETP and oily water treatment plant) or at the discharge of P-007 A/B/C/D/E/F/G. As per ITB requirement same is considered at the UF outlet from ETP. Client to confirm. |  |  |  | Consider dosing of oxygen scavenger at the outlet of UF (of ETP and oily water treatment plant). |  |  |
|  |  | Technical |  | - |  |  | - |  |  | - |  |  | Sizing of CBD |  |  | Please note that to limit the capacity of CBD during blocked outlet case (wet crude oil to wet crude oil tank XSV fail close); it is required to ensure the high probability of closure of XSV-2001 and XSV-2002 on high pressure in wet crude oil header. Same can be achieved through 2oo3 logic of high pressure of wet crude oil header signal which initiate the closure of XSV-2001 and XSV-2002. With this the residence time of CBD can be considered as low as 1 minute. BIDDER understands that the SIL level of above shutdown loop as SIL-2 shall be acceptable. |  |  |  | SIL 2 is acceptable. |  |  |
|  |  | Technical |  |  |  |  | TQ Replies |  |  | TRS-05 (Sr No 01) |  |  | BOD/COD |  |  | As per TRS-05, Sr No 01, Owner specified, “Please consider the BOD as 80-135 mg/l and COD as 180-1400 mg/l in the inlet of effluent treatment plant. There is no requirement of maintaining BOD & COD in treated water from ETP. However, in order to meet desired BOD/COD parameter at the inlet of UF, the bidder is free to suggest his own scheme.” Based on the vendor feedback, there is some limitation of COD value at UF inlet approximately (100-150 ppm), for treatment of given rage of COD (180-1400 mg/l), a biological system is required before feeding water to UF system. The Biological system may include Aeration Tank, Clarifier, Blower, Dosing system etc. Request OWNER to review and confirm any specific requirement of consideration of such a high COD (1400 ppm) at the inlet of ETP. |  |  |  | Kindly follow TRS-05 response. If anything required to be done for safety of the equipment / system same shall be considered by bidder. |  |  |
|  |  | Technical |  | Section 2- Mechanical _Section 2.1_Doc No - TCE.7547A-H-OST-300.(revised specification for RWTP) |  |  | General Requirement for Water Treatment and Water Supply |  |  | 2.2, 2.3 |  |  | Industrial and Potable Water Quality – TSS |  |  | Based on OWNER response vide TRS no 4, Process, Sr. no – 1, there is no requirement of Ultrafiltration for potable water. As per specification TSS in industrial water shall be less than 10 ppm whereas for potable water TSS shall be NIL. Considering only 5 micron activated carbon cartridge filter and UV on potable water lines, TSS in potable water as NIL may be difficult to achieve. Request OWNER to confirm any additional system to be considered to meet potable water TSS as NIL. |  |  |  | Tender specification says about quality of water required for potable water .Tender indicates the minimum requirements of system. If vendor feels the requirement to add additional equipments in the system to meet the guaranteed parameters, they may do so. However, specifications of the potable water shall be met as per tender. |  |  |
|  |  | Technical |  |  |  |  | TQ Replies |  |  | TRS-03 (Sr No 31 Process) |  |  | Pump Flow Control for Pump P-007A/B/C/D/E/G |  |  | Against the BIDDER query of minimum flow control for P-007, OWNER reply was “Flowrate for injection well station is 50m3/hr. SOPs shall be developed by bidder.” With the above reply, BIDDER has following understanding 1) There is no requirement of pump min. flow rate as 10 m3/hr. 2) Pump normal and rated flow rate will be 50 m3/hr. 3) Since no requirement of min. flow rate, the motor to be considered as fixed speed and speed reducer is not to be considered for the said pump. Request OWNER to confirm understanding. |  |  |  | - Requirement of pump min. flow rate shall be decided by Pump Vendor / successful bidder. - Refer reply to point no. 1) above. - Refer reply to point no. 1) above. |  |  |
|  |  | Technical |  | Section 2- Mechanical _Section 2.1_Doc No - TCE.7547A-H-OST-300.(revised specification for RWTP) |  |  | General Requirement for Water Treatment and Water Supply |  |  | 2.4, 4 |  |  | Boiler Feed Water |  |  | As per Boiler Feed Water specification, TDS shall be NIL. Based on RO vendor feedback, to meet TDS as NIL will be difficult. Further as per IS 10392-1982, higher TDS values can be acceptable. The boiler vendors consulted by Bidder have confirmed a TDS of 10 ppm as acceptable. BIDDER understand that there no specific requirement to meet Boiler Feed Water TDS performance parameter as NIL unless there is specific requirement from boiler vendor. OWNER to confirm BIDDER’s understanding. |  |  |  | Boiler feed water quality from RO shall meet the requirements as per IS10392-1982 and Boiler manufacturer. |  |  |
|  |  | Technical |  | Section-1_Process_Subsection_1.9.4_Process Pumps (TCE.7547A-300-PDS-8708) |  |  | Process Datasheet for Dry Crude Oil Pumps |  |  | 22 |  |  | Minimum Flow |  |  | As per P-003 datasheet, the normal and rated capacity of the pump is 140 m3/hr. The minimum capacity is indicated as * i.e. bidder to specify. As per Process Design Basis (TCE.7547A-CH-300-DB-01) clause-9 for Dry Crude Oil Dispatch Pump the minimum flow of P-003 is 28 m3/hr. Since the purpose of P-003A/B/C/D is to transfer the dry crude oil from STF to NHK CTF, BIDDER understands that there is no specific requirement of controlling the pump flow rate at 28 m3/hr. The minimum possible flow can be decided by the vendor and same can be acceptable. Kindly confirm the above understanding. |  |  |  | Confirmed. |  |  |
|  |  | Technical |  | - |  |  | TQ replies |  |  | TRS – 02, Process, Sr. no-42, |  |  | Continuous Emission Monitoring System and Emission Monitoring system. |  |  | As per OIL reply against CEMS requirement vide TRS-02 (Process, Sr. no-42), COMPANY confirmed the following: - Continuous Emission Monitoring through NOx & CO analyzers for Boiler and CPP. - Bidder to comply as per specification for P-003A/B/C/D, BH-001A/B, BH-002A/B/C/D and TP-01/02/03. Based on the above reply from the COMPANY, BIDDER has following understanding. COMPANY to confirm BIDDER’S Understanding. - Continuous Emission Monitoring System (Analyzers for NOx, CO) to be considered for CPP and Boiler only. - Only Emission Monitoring System to be considered for P-003A/B/C/D, BH-001A/B, BH-002A/B/C/D and TP-01/02/03. Emission monitoring system will have only sampling ports on Flue Gas Stack for Manual Sampling. There will not be any analyzers for online reading. - Sampling through sampling ports for the above mentioned equipment’s (Sr. no-2) shall be outside EPC contractor’s scope. Requirement of portable analyzers for the above mentioned equipment’s (Sr. no-2) shall be outside EPC contractor’s scope. |  |  |  | - Confirmed - Vendor shall refer applicable statutory requirements / rules / regulations indicating requirement of Continuous Emission Monitoring System for the process equipment / systems form part of STF. - Confirmed - Refer reply to point no. 2 above. - Refer reply to point no. 2 above. |  |  |
|  |  | Technical |  | - |  |  | - |  |  | - |  |  | CEMS |  |  | As per CPCB emission norms (Furnace, Boiler and Captive Power Plant)- i) All the furnaces/boilers with heat input of 10 million kilo calories/hour or more shall have continuous systems for monitoring of SO2, PM, CO and NO2. Manual monitoring for all the emission parameters in such furnaces or boilers shall be carried out once in two months. ii) All the emission parameters in furnaces/boilers having heat input less than 10 million kilo calories/hour will be monitored once in three months. COMPANY to confirm the following. - Heat input to Boiler and CPP is expected to be less than 10 million kcal/hr. If so, BIDDER can delete the requirement of CEMS (NOx, CO analyzers) for Boiler & CPP by providing manual sampling ports only. COMPANY to confirm BIDDER’S understanding. - As per CPCB, there is requirement of Particulate Matter (PM) analyzers. COMPANY to confirm whether PM analyzers to be considered or not for Bidding purpose. - For CEMS, Oxygen analyzers are required to measure the Oxygen content in Flue gas. However Oxygen analyzer is only considered for Boiler. COMPANY to confirm the requirement of Oxygen analyzer for CPP. - Heat input to P-003A/B/C/D, BH-001A/B, BH-002A/B/C/D and TP-01/02/03 is expected to be less than 10 million kcal/hr. Hence, there is no requirement of CEMS for these packages. However, sampling ports to be provided for manual sampling as per CPCB norms. COMPANY to confirm BIDDER’S understanding. |  |  |  | - To provide CEMS as per tender - For NG boiler PM is not required. - O2 analyser not required for CPP as per tender. - Heat Inputs to Gas Engines (P-003 A/B/C/D) shall be calculated by bidder & accordingly necessary instruments / systems shall be evaluated as per statutory requirements / rules / regulations. Note- sampling port to be provided in boiler and CPP stack in addition to CEMS. |  |  |
|  |  | Technical |  | PART-II, Technical Section-I, Process, Process Design Basis (TCE.7547A-CH-300-DB-01) |  |  | Process Design Basis |  |  | 29.4.5 |  |  | PHE cleaning system |  |  | Process Design Basis clause 29.4.5 states that “Chemical clean-out connections, if required, will be provided on the required side of the exchanger, between the exchanger and isolation blind should a spool piece be required.” Accordingly BIDDER has considered 2” connection nozzles for PHEs. However in line with ITB P&IDs, BIDDER has not considered PHE bypass / backwash arrangement / CIP (PHE cleaning system). COMPANY to confirm BIDDER’s understanding. |  |  |  | Requirement of PHE bypass / backwash arrangement / CIP (PHE cleaning system) shall be decided by bidder based on the inputs from Heat Exchanger vendor. Further, refer note no. 6 on PHE Process Datasheets |  |  |
|  |  | Technical |  | Part-II Technical section, Section-I, PROCESS, (Doc No - TCE.7547A-H-OST-300, ) (Doc No -TCE.7547A-300-PDS-8704, Rev R0) (Doc No -TCE.7547A-300-PDS-8705,Rev R0) |  |  | Write up, Page no 2 of 13. PDS for Electrostatic Emulsion Treater PDS for Indirect Bath Heater |  |  | 3.2 - - |  |  | Free Water & Emulsified Water content in feed to BH-002A/B/C/D |  |  | OWNER reply against the BIDDER query regarding Free Water & Emulsified Water content in feed to BH-002A/B/C/D (Process -Sr. No. 43 in TRS2) is “Shall be finalized by bidder based on inputs form respective vendor. Dehydration unit (EET+BH) shall perform the desired duty as per tender specifications.” OWNER to please note that with consideration of separation of 40% free water in wet crude oil tank, the feed to BH-002A/B/C/D will be (A) Flow = 13000 KLPD (Design) Crude oil content = 80-100 % v/v BS&W content = ~ 0-20 % v/v Free water = NIL Emulsified water = ~ 20 % v/v However the above data as per BID EET datasheet is as follows – (B) Flow = 13000 KLPD (Design) Crude oil content = 50-100 % v/v BS&W content = 0-50 % v/v Free water = 40 % v/v Emulsified water = 10% v/v Please note that the above two options (A) and (B) shall have an impact on the sizing of Dehydration unit. Since the tender specification (i.e. Process datasheet of B-002A/B/C/D, EET-001A/B/C/D) provides incorrect design values, same cannot be considered for design of dehydration unit. Request OWNER to specifically confirm which option (Option A or Option B) values to be considered for design of dehydration unit. |  |  |  | Kindly note that the unit has to function at design case of 13000 kLPD. Datasheets considered worst case for Dehydration units design. Follow the datasheet values. Emulsified water content shall be considered as 10% of total flow for Dehydration unit. |  |  |
|  |  | Technical |  | Part - II: Technical Section, Section -II- Mechanical (Spec No - TCE.M4-127-06, Rev R2, |  |  | Section 2.3 -Cooling Tower Datasheet |  |  | - |  |  | Cooling range |  |  | As per CTI ATC 105, cooling range required cooling range is 40C ± 20%. As per cooling tower datasheet, cooling range specified is 40C ± 20C. BIDDER understands that cooling range of 40C ± 20% can be acceptable as per CTI ATC 105. OWNER to confirm BIDDER’s understanding. |  |  |  | Bidder query it seems to be typo graphical error, instead of 4 Deg C mentioned as 40C range. As per CTI ATC 105 says, Range +/- 20%. As per tender, Range mentioned as 4 Deg C+/-2 Deg C which will cover in the above mentioned std. Bidder to comply as per tender datasheet. |  |  |
|  |  | Technical |  |  |  |  |  |  |  |  |  |  | Dosing chemical tank capacity |  |  | The size of the individual dosing tank depends on the number of days storage capacity of chemicals as required by Client. Client to provide the storage capacity (in days) for dosing chemicals. |  |  |  | Kindly consider 7 days storage. |  |  |
|  |  | Technical |  |  |  |  |  |  |  |  |  |  | Chemical dosing - De-emulsifying agent |  |  | Client to note that the dosing rate of de-emulsifying additive depends on the results of the field test and bottle test of the crude. The same are not available along with the bid document. However bidder will go ahead with some assumption based on his past project experience and same shall be confirmed by Client during post bid stage. Client to confirm the understanding. |  |  |  | Kindly note that field test and bottle test results will be shared with successful bidder. |  |  |
| HSE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  | - TCE.7547A-H-OST-300 - OISD-117 - TCE.7547A-CH-300-DB-01 |  | Fire Protection System |  |  |  |  |  | - 2.8 |  |  | Fixed Foam System and Automatic Rim Seal Fire Protection System |  | As per OISD-117 Clause 4.2.12, Automatic actuated Rim Seal fire detection and extinguishing system shall be provided on external floating roof tank storing Class A Petroleum products only. As per Process Design Basis and referring to the flash point of Wet Crude Oil, the petroleum class for this fluid is Class-B Therefore, bidder understands that as per OISD-117 (for Petroleum Class-B fluid), Automatic actuated rim seal foam system is not required for the wet and dry crude oil storage tanks. However, fixed foam pourers for rim seal area are considered by bidder. COMPANY to confirm Bidders understanding. |  |  |  |  | Bidder to comply tender specification. |  |
| 2 |  |  | - M. B. Lal Recommendations |  | IOC fire accident investigation report by M.B. Lal |  |  |  |  |  | 10.6 (Page No. 10 of 23) |  |  | Fire Fighting Facilities |  | ITB does not mention about Deluge Valve requirement for Fire Water & Foam System. However, as per M. B. Lal recommendations it is recommended that “auto start of the fire water pumps to be linked with the Hydro Carbon Leak Detection and Alarm System in order to start the sprinkler system automatically especially in tank farm and pump house.” Therefore bidder is considering the deluge valves for Firewater and Manual Valves for Foam System. Company to kindly confirm bidders understanding. |  |  |  |  | Bidder understanding is correct |  |
| 3 |  |  | SPEC NO. 7547A-HOST- 300 |  | Gas Engine, Design Basis for Gas Engine Generator Set |  |  |  |  |  | 2.1.4 (Page 2 of 9) |  |  | F&G detector provision and their action implication |  | Since separate acoustic enclosures for individual GENSET is not envisaged and same is confirmed by Company vide earlier TQ reply, bidder is proceeding with the provision of F&G Detectors inside the CPP building. These detectors and their action shall be common for all GENSET (i.e. on confirmed gas detection inside the CPP, the action shall be to trip all GENSET). This will completely stop the supply of emergency power to the entire facility. Company to confirm bidders understanding. |  |  |  |  | Kindly refer to TRS-1 Mechanical Sl. No 7. To note Individual Gensets shall have individual enclosures. They shall come with individual F&G detectors. F&G detectors are considered already for CPP building please refer instrumentation tender document – F&G Layout in Annexure 4.6 |  |
| 4 |  |  | TCE.7547A-300-GA-8045 Rev P0 |  | Hazardous area classification layout- Plan secondary tank farm |  |  |  |  |  |  |  |  | Hazardous Area Classification for Circulation Pump House (Wet & Dry Crude) |  | The hazardous area classification layout indicates that the Dry Crude & Wet Crude Oil Circulation Pump House (Pump + Gas Engine) to be classified for Zone 1. However, since the source is under secondary grade of release, as per OISD-113 Clause No. 5.2.3, this area should be classified as Zone-2. Therefore, Bidder is considering the Dry Crude & Wet Crude Oil Circulation Pump House (Pump + Gas Engine) as Zone 2. COMPANY to confirm the Bidders understanding. |  |  |  |  | Kindly follow the tender documents. |  |
| 5 |  | Technical | TCE.7547A-HOST- 300 |  | Scope Of Work And Design Specification |  |  |  |  |  | - |  |  | Process & Non-Process Areas |  | As per Note-10 of the Hazardous Area Classification Drawing, it is indicated that all the equipment within Process Areas shall be selected for Hazardous Area (Zone-2) irrespective of the area being unclassified or safe. However, there are no clear guidelines for segregation of the Process and Non-Process Areas in any of the ITB documents. Company is requested to provide the segregation of process and non process areas As per present understanding of the Bidder the following Equipment/Packages are considered as Process and Non-Process Areas:- Process Area: - Preheater & Dehydrator Packages - Dry Crude Oil Dispatch Pump House - Wet Crude Oil Circulation Pumps - Crude Oil Tanks including Bund - Chemical Dosing Package for Crude Oil Non-Process Areas:- - Fire Water System - Boiler - Instrument Air Compressor Package - CPP - Foam Pumps - Raw Water Treatment - Effluent Treatment Package - Clarified water pump house - Sewage Treatment Package - Sludge Treatment package - Raw Water Treatment Package - Cooling Tower Package+ Heat Exchanger Company is requested to confirm Bidder’s understanding or advise otherwise. |  |  |  |  | Kindly consider oily water handling equipment i.e. API, TPI etc. of ETP as Process area. |  |
| 6 |  |  | - SPEC. NO. TCE.7547A-H-OST-300 - P&ID No. TCE.7547A-617-PI-1106) |  | FIRE PROTECTION SYSTEM |  |  |  |  |  | 2.2 (Page No. 4 of 14) |  |  | Diesel Engine Fuel Tank Capacity for Fire Water Pump |  | As per the document (TCE.7547A-H-OST-300) Each engine shall have an independent fuel tank adequately sized for 6 hours of continuous running of the pump. FW P&ID (TCE.7547A-617-PI-1106) mention that “Diesel Engine Day Tanks for 24 hours”. Bidder understands that the fuel tank for diesel engine should be sized for 6 hours of continuous running but the fuel should be available for 24 hours. (i.e after 6 hours of operations the tank should be replenished with diesel fuel with no time lag) Company to kindly confirm bidders understanding. |  |  |  |  | Bidder understanding is correct except that the Tank size shall hold fuel for atleast 8 hours continuous running (in place of 6 hours) of the Pump at full load. |  |
| ME-S |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | Technical | TCE-7547A-H-OST-300 |  |  |  |  |  | Gas fired Package Boiler Specifications |  | 1.1.6 |  |  |  | Cladding for Boiler Shell insulation | The said clause in bid documents calls for SS outer clad material for boiler shell The normal practice is to provide the Aluminum cladding for insulation. Kindly confirm the acceptability of Al cladding. |  | Shall be as per tender. |  |  |  |  |
| INSTRUMENTATION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  | SOW TCE 7547A-H-OST-300 ,clause 4.4.7- |  |  |  |  | TQ reply TRS1 |  |  | 22 |  |  |  |  | As per TQ reply “Rim seal protection communication from STF control room to new building in duliajan, cabling is by others”, Bidder understands that all civil works, termination required for laying of cables is also by others. Tenderer to confirm the understanding of bidder |  |  | Refer Reply Elsewhere. |  |  |  |
| 2 |  |  | SPEC NO.7547A-H-OST-300, Section 2.3, sheet no.2/9, |  |  |  |  |  |  |  | 1.22 |  |  |  | CEMS & AAQMS on line data transmission to CPCB | As per CPCB norms data transmission is via internet using Leased Line of 01 MBPS capacity. Bidder understands that Leased line of 01 MBPS will be provided by Tenderer at Central Control Building. Tenderer to confirm the understanding of bidder. |  |  | The same is in bidder’s scope. |  |  |  |
| 3 |  |  | TCE.7547A-OST-300 |  |  |  |  | AMBIENT AIR QUALITY MONITORING |  |  | Note no 6 |  |  |  |  | As per the note separate UPS to be provided for AAQMS, Bidder has considered independent feeder from main UPS instead of separate UPS which is to be placed inside the AAQMS shelter. Tenderer to confirm the understanding |  |  | Please proceed with independent feeder from main UPS. |  |  |  |
| 4 |  | CEMS | TCE.7547A-H-OST-300 |  |  |  |  |  |  |  | 3.1.4.m |  |  |  | CEMS for packages | ‘Shelter room location for ambient monitoring system shall be finalized by EPC contractor as per Pollution Control Board (PCB) norms.’, Clause is clear for AAQMS but we understand that for CEMS in different packages shall have Free stand-alone panel suitable for area classification Company to confirm the understanding |  |  | Confirmed. |  |  |  |
| 5 |  | CEMS |  |  |  |  |  |  |  |  |  |  |  |  | CEMS | Please provide us, specification and Datasheets for CEMS ,Oxygen analyser, Analyser Cabinet/Analyser Shelter |  |  | - Vendor shall refer applicable statutory requirements / rules / regulations indicating requirement of Continuous Emission Monitoring System for the process equipment / systems form part of STF. |  |  |  |
| 6 |  | CEMS |  |  |  |  |  |  |  |  |  |  |  |  | CEMS | We understand CEMS requirement for packages as below 1. Captive Power Plant:4 nos CEMS 2. Dehydration Unit Package(BH-002 A/B/C/D): 4 nos CEMS & 4 nos O2 analyser 3.Preheater Package (BH-001 A/B) :2 nos CEMS & 2 nos O2 analyser 4. Boiler and Auxiliaries Package: 1 nos CEMS & 2 nos O2 analyser We also understand for each CEMS analyzer cabinet suitable for area classification is to be considered Company to confirm the understanding also please provide general specification for Analyser cabinet |  |  |  |  |  |  |
| 7 |  | CEMS |  |  |  |  |  |  |  |  |  |  |  |  | CEMS | Tenderer to provide details of Existing system/software used by OIL to transfer CEMS data to CPCB/APCB |  |  | New system By bidder |  |  |  |
| CIVIL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Clarification |  | TCE.7547A300-PP-1000 |  |  | Plot plan |  |  |  |  |  |  |  |  |  |  | There are two sheds shown on the plot plan. It is not clear whether these sheds are to be retained or dismantled. It is presumed that these sheds are to be retained. Company to kindly confirm our understanding. Otherwise company is requested to provide clarity on scope of demolition & related As-built drawings, for estimation of Demolition quantity. |  |  |  | The existing two sheds shall be retained |  |
| 2 | Construction |  | TCE.7547A-HOST- 300 |  |  | ENGINEERING & ARCHITECTURAL DESIGN BASIS REPORT |  |  |  |  | 9.25 | Landscaping and Green Belt Development |  |  |  |  | Only following clauses were found in ITB, which do not give information about what is expected for landscaping / green belt development: - "Civil 1.pdf" - section 5.1 - cl. 1.2.45 - "Civil 4.pdf" - cl. 1.1 - "Civil 11.pdf" - section 5.7 - cl. 9.25 Company is requested to kindly provide clear information about the same, to enable us to include in costing. Please provide us detailed specification, BOQ & Preliminary landscape layout drawing in order to cost us for company’s requirements. |  |  |  | Landscaping shall be done as per local regulatory norms, MOEF / other statutory requirements and shall include & not limited to Green belt and landscaping area indicated in plot plan. The scope shall include planning, supply, installation and maintenance of approved plants, necessary hardscape works, softscape works and irrigation systems. The landscaping shall be carefully studied and include but not limited to the following: Providing high trees by the road or walkways wherever necessary. Planting Trees, Plants and shrubs in the green belt area. Planting material list shall be submitted for approval by EMPC/OIL. The Handing over of the Green-Belt area shall be along with handing over of the complete facility to the O&M Contractor / OIL as applicable. |  |
| 3 | Furniture |  | TCE.7547A-HOST- 300 Civil – 1: Section 5.1 Section 5.3 |  |  | - Scope of Civil, Structural and Architectural Works, - Technical Specification for Architectural work |  |  |  |  | Cl. No. 1.2.46 Cl. No. 10 | Furniture BOQ/MTO |  |  |  |  | Specification for furniture requirement is given in the ITB. However, MTO/BOQ is not provided. We will not be in a position to cost for the same in absence of BOQ. Bidder requests client to provide the BOQ for furniture. |  |  |  | Tentative BOQ for furniture is provided herewith. |  |
| 4 | Construction |  | TCE.7547A-HOST- 300 |  |  | ENGINEERING & ARCHITECTURAL DESIGN BASIS REPORT |  |  |  |  | 9.25 | Landscaping and Green Belt Development |  |  |  |  | In order to ensure draining of storm water, bidder understands that landscape area also needs to be back filled. Bidder understands that top soil removal is not required and back filling will be done on top soil for landscaping areas. Company to confirm |  |  |  | Top soil removal shall be done for the entire site. Bidder to follow the tender document. |  |

| ME-R |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | TECHNICAL | TCE.M4-105-01 | 1. Diesel Engine & Accessories Data Sheet– A | 1. Doc No. TCE. 7547A-H-OST-300 SEC No. 2.5.4, 1. Sr. No. 39, pg no. 4 of 4 | UL / FM APPROVAL | Bidder understands that the fire water pump, diesel engine and its controller shall be as per the requirements of OISD / NFPA 20. However, no UL / FM listing is required for these equipment. | Bidder to comply tender specification. |
| 2 | TECHNICAL | TCE.M4-132-02: | Diesel Engine Data sheet | Sr. No. 9.0 | Noise Level | Bidder understands that noise level of 85 db(A) at 1 m is not applicable for diesel engine driven Fire water pump. Also bidder understand that there is no requirement of acoustic enclosure for Fire Water Pump package. Company to confirm. | Bidders understanding is correct. |
| 3 | TECHNICAL | SPEC NO.7547A-H-OST-300, Section 2.3, | Captive Power Plant | sheet no.5/9, Clause 3.4.3 | No of starts for batteries | Company has confirmed 3 No of Starts in response to query listed in sr no. 33 of TRS-1. Also Company has informed that Maintenance free batteries, with capacity suitable for minimum six (6) starts in response to query sr no. 59 of TRS-1 As vendors have quoted battery rated for 3 starts as std design, Bidder request Company to confirm that 3 nos. consecutive starts shall be acceptable for CPP Genset . | Shall be as per TRS-1 |
| 4 | TECHNICAL | SPECS NO. 7547A-H-OST-300 | Captive Power Plant - Gas Engine | SR NO. 4.4 | Noise level | At two places observed two different responses for requirement of acoustic enclosure Query sr no:- 23, TRS-3 Query sr no:- 7, TRS-1 ( Query sr no. 23, TRS-3 ) In the adjacent clause it is mentioned that acoustic enclosure is required for gas engine pump and for CPP genset noise would be measured outside the building. (Excel sr no;- 7,TRS-1) In this clause , Oil India responded that acoustic enclosure is required for individual CPP genset. Bidder understands that acoustic enclosure is not required for individual CPP gensets and only CPP building will be acoustically treated to maintain 85 dBA at 1 m on outside of the building. Company to confirm. | Acoustic enclosure is required for individual CPP gensets to maintain 85 dbA individually, and CPP building shall also be acoustically treated to maintain 85 dBA at 1 m on outside of the building with all gensets running in acoustic enclosure open condition. |

| 5 | TECHNICAL | 1) TCE.7547A-300-GA-8045 Rev. P0 2) TCE.7547A-H-OST- 300, Rev R0) |  | 1) TCE.7547A-300-GA-8045 Rev. P0 | Hazardous Area Classification Layout has not provided with classification of entire facility. | As per vendor offers, while PD pump side will be considered as hazardous area zone 1, but Gas Engine side shall be considered as safe area as it is separated by brick wall. Accordingly, all instrumentation and batteries etc for Gas Engine will be suitable for safe area. Company to confirm. | Gas Engine Instrumentation shall be suitable for Zone – 2. |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 6 | TECHNICAL | 7547A-HOST- 300 | Gas Engine for Pumps | clause no. 2.4 (sheet 2 of 8) | Driver Sizing | Clause 2.4 states that "The continuous net engine brake horse power rating (after accounting for all auxiliary power consumption) at the site condition shall be at least 20% greater than brake horse power required to drive the pump at its duty point at the rated rpm and in no case less than the brake horse power required to drive the pump at 150% of rated discharge or at any condition of operation of pump". Bidder understands that the Gas Engine driver rating shall be 20 % more than the power required to drive the pump at the rated duty parameters of 140 m3/hr flow and 25.1 kg/cm2 g discharge pressure. The condition of driver sizing for 150% of rated discharge shall not be applicable. Company to confirm. | Refer corrigendum |
| 7 | TECHNICAL | TCE.7547A-300-PI-8035 | P&ID OF DOSING SYSTEM | SH 01 of 01 | Missing Specifications | Company confirmed that VFD is not required, hence bidder understands that capacity control for dosing pumps for Chemical Dosing Packages shall be by automatic stroke length adjustment only. | Kindly refer response to Process query no. 2. |
| 8 | TECHNICAL | TCE.7547A-300-PDS-8708 | DATA SHEET FOR RECIPROCATING PUMP (Dry Crude Oil) | Row-17 (SHT 1 of 2) | Solid content in dry crude oil | Bidder understands that Dry Crude Oil will have some solid content. Bidder requests Company to provide the solid particle size & content to be considered for PD pump (P-003A/B/C/D) | Dry Crude may contain some wax particles. No solids are expected. |
| 9 | TECHNICAL |  | A-Design Basis PFD | Clause 6.1.13 | Sludge disposal | For Sludge Handling for first 3 months by Bidder, Company to Provide details for sludge disposal, viz location, mode of sludge transfer & any other statutory permissions and Norms required to be met for the same. | Location will be informed to successful bidder. Rest requirement shall be finalized by bidder. |
| 10 | General | TCE.7547A-H-OST- 300 | SPARES | Clause No 1 | Spare List | Query "Document TCE.M4-907 mentioned in said clause was not found in ITB." This Document is not found in corrigendum to ITB. Company to furnish the same. | Kindly ignore this document. |
| 11 | Technical | TCE.7547A-300-PDS-8708 | DATA SHEET FOR RECIPROCATING PUMP |  | Net Positive Suction Head Available (NPSHA) | Due to very high length of suction pipe and header length, the acceleration head losses will be very high and it will affect the Net Positive Suction Head Available (NPSHA) for the pump. So bidder understands that a surge vessel may have to be added based on acceleration head calculation. Company to kindly confirm bidders understanding. | Surge vessel is not envisaged. If, the systems demands, same has to be decided by successful bidder. Bidder to ensure the process requirement is met. |
| ELECTRICAL |  |  |  |  |  |  |  |
| 1 | Technical | - | Technical Queries- Replies | 1. TRS-2, EL-TQ-2 2. TRS-3, EL-TQ-1 | CGGS Cable Routing | With respect to the referred Bidder’s queries Company has responded that the CGGS is located at an approximate distance of about 600mtrs from the STF substation for consideration of UPS Incomer Cable. However, the following details are not available with the Bidder:- - The UPS Cable route from CGGS to STF substation. - Whether the cable shall be laid in any existing tray/trench or new trays/trenches shall be installed. - If existing trays/trenches are required then space available for the same. - If new trays/trenches are required then whether the same is in bidder’s scope of work. Company is requested to provide the above data to the Bidder. | Buried Trenching including associated jobs for Cable is in Bidders scope. To consider approx. 600 meters for Bidding purpose. |
| 2 | Technical | TCE.7547A-HOST- 3002, SECTION: 3.1 | Scope Of Work And Design Specification | Cl.no.2.2.7 | Soft Starter Requirement | As per the referred clause all motors rated above 7.5kW shall be fed from Soft starter. - Company is requested to clarify the reason for providing Soft Starters for all Motors above 7.5kW. - As per Bidder’s understanding, the requirement of soft starter shall be based on System Study Calculations due to the following complications associated with such large no. of soft starters:- - Injection of excessive Harmonics in the Electrical Network and need for compensation of the same. - Reduction in overall power factor which is turn will have an adverse effect on the Gas Gensets. - Additional Space for MCCs due to larger dimensions. Company is requested to confirm above Bidder’s understanding | Request bidder to comply as per specification. |
| 3 | Technical | TCE.7547A-HOST- 3002, SECTION D-3.23 | Mandatory Spares | Cl.no.10 | Motor Mandatory Spares | - It is observed that “Complete stator core pack with winding” has been indicated under the Motor mandatory spares. As per Bidder understanding based on past executed project experience, the same is not required to be included in the Mandatory spares. - Also the Mandatory Spare list for the Motors is not applicable for any of the Engine Auxiliary motors. Company is requested to confirm bidder’s understanding or advise otherwise. | Bidder to comply as per Specification |
| 4 | Technical | TCE.7547A-HOST- 300 SECTION: 3.1 | SCOPE OF WORK AND DESIGN SPECIFICATION | 2.10.11 | Lighting | Under plant lighting system, as per cl. no. 2.10.11 Peripheral Lighting shall be provided using Double Pole structures of required heights. Company to note the standard installation drawing for the double pole structure and the type of lighting fixtures for the same is not indicated in the ITB. Company is requested to provide the above data for periphery lighting to the Bidder. | Bidder to employ the standard installation practices in drawings (TCE.M2.EL - GL-S-2219) attached with Tender Specification. |
| 5 | Technical | TCE.7547A-HOST- 3002, SECTION D-3.23 | Mandatory Spares | Cl.no.10 | Alternator Mandatory Spares | It is observed that Mandatory Spares for the Alternator are not available in the referred document or any other ITB document. Company is requested to provide the same. | Bidder to refer Section -6 General |
| 6 | Technical | - | Technical Queries- Replies | TRS-3, EL-TQ-4 | VFD & Soft Starter Specifications | With respect to the referred Bidder’s queries Company responded that since VFD are not required, the specifications for the same are not provided whereas nothing is mentioned about the Soft starter specifications. Company is requested to note that VFD requirement is indicated in various Process documents like P&ID, PFD, etc. In the absence of the VFD & Soft starter specifications Bidder understands that the above equipment shall be as per Manufacturer specifications. Company is requested to confirm bidder’s understanding or provide the specifications/datasheets for VFDs/Soft starters. | As per Clause no:2, process above the VFD is accepted only for ETP. Bidder to refer corrigendum for VFD specification. The technical details of soft starter shall be furnished for our approval during detail engineering stage. |
| 7 | Technical | - | TCE-7547A-SK-580-001 | - | - | As per the System Architecture drawing received with the Corrigendum, an “Upcoming Fire Station at Duliajan” is indicated. Bidder understands that the above mentioned upcoming Fire station and FO connection to the same is not in the EPC contractor scope. Company is requested to confirm Bidder’s understanding or advise otherwise. | Please find the reply elsewhere. Bidder`s understating is correct. |
| 8 | Technical | - | Technical Queries- Replies | TRS-3, EL-TQ-8 | Field MCP & Hooter Connection | Company has responded to Bidder’s query that Conventional MCPs & Hooters shall be installed in Field. Bidder understands that the Conventional MCP & Hooters shall be connected to the F&G system. Company is requested to confirm bidder’s understanding or advise otherwise. | Confirmed |